GRC Façade Inspection Checklist

Use this checklist as a printable or digital aid when inspecting and maintaining glass reinforced concrete (GRC/GFRC) cladding systems. Adapt frequencies to your asset-management plan.

1. Pre-task planning

- [ ] Method statement prepared and approved

- [ ] Task briefing completed with all operatives

- [ ] Lifting plan and access strategy defined

- [ ] Required permits obtained from asset/station operator

- [ ] Hoarding / barriers planned where works occur near the public

2. Access and safety

- [ ] Appropriate access equipment selected (MEWP, scaffold tower, lifting robot, mini crane)

- [ ] Access equipment inspected and within certification dates

- [ ] Working-at-height training verified for all operatives

- [ ] Fall-protection and safety tethers available where required

- [ ] Escalators or sensitive assets protected before tower erection

3. Routine visual inspection of GRC panels

- [ ] All visible panels checked for looseness or insecurity

- [ ] Panels checked for misalignment or displacement

- [ ] Surface staining, grease or oil contamination identified

- [ ] Dust build-up and general soiling recorded

- [ ] Graffiti or impact damage noted

- [ ] Interfaces with adjacent architectural elements inspected

4. Substructure, fixings and anchors

- [ ] Steel substructure inspected for deformation or damage

- [ ] Brackets, rails and stud sections checked for corrosion

- [ ] Mechanical fixings (bolts, screws, connectors) checked for looseness

- [ ] Safety tethers and locking mechanisms inspected

- [ ] Concrete anchors visually inspected where accessible

- [ ] Anchor pull-out tests carried out where specified

5. Hidden components: insulation and smoke barriers

- [ ] Acoustic insulation condition checked where panels are removed

- [ ] Horizontal smoke barriers inspected and intact

- [ ] Vertical smoke barriers inspected and intact

- [ ] Any damaged or missing barriers scheduled for repair/replacement

6. Cleaning operations

- [ ] Cleaning method selected (hand-wipe, brush, pole system)

- [ ] Only approved cleaning agents used (e.g. lukewarm soapy water)

- [ ] Bleaches, abrasive cloths or harsh detergents explicitly excluded

- [ ] Passenger-interface panels cleaned at agreed frequency

- [ ] High-level panels cleaned according to site-specific needs

- [ ] Deep-clean schedule (every 2–5 years) reviewed and updated

7. Panel repair and replacement

- [ ] Minor chips and edge defects assessed for repair

- [ ] Repair mix / kit available and compatible with GRC system

- [ ] Perforations or holes patched using appropriate tools

- [ ] Sanding and finishing carried out to acceptable visual standard

- [ ] Panels scheduled for removal/replacement identified

- [ ] Removal and reinstallation procedures followed step-by-step

8. Escalator and complex geometries

- [ ] Special access arrangements planned for escalator shafts and tight spaces

- [ ] Panels in complex geometries inspected for movement and damage

- [ ] Any bespoke fixings or brackets checked and documented

9. Testing and QA

- [ ] Torque checks completed on critical screws and bolts

- [ ] Test results recorded against asset IDs or panel references

- [ ] Inspection and test plans (ITPs) updated where applicable

10. Documentation and reporting

- [ ] Inspection findings recorded in a written report

- [ ] Photos taken of defects, repairs and critical components

- [ ] Outstanding actions assigned with deadlines and responsibilities

- [ ] Maintenance history updated in asset-management system